To send command to the ILT1000, you need to send the first character, wait 50ms, then send the remaining characters. You can do this in a couple of ways:

1. Send the characters 1 by 1 and just leave a 50ms delay between each. This is how the example Labview code do to simplify the block diagram

2. Send the characters 1 by 1, leave a 50ms delay after the first characters, then send the others without delay.

Others notes:

1. \r needs to be the only trailing characters to indicate the end of the command

2. On receipt, read until you get a \n

For this device, the only setup should be for serial port: 115200,N,8,1,no HW or SW handshakes.